Eddy currents in a ground plane are circulating currents induced in conductive planes by time-varying magnetic fields. From an engineering perspective, they are neither inherently good nor bad—but uncontrolled eddy currents create loss, noise, and unintended coupling.

1. Physical origin (Maxwell-level view)

Any changing magnetic flux dΦdtthrough a conductive loop induces a voltage:

∮E⋅dl=-dΦdt

A solid ground plane provides many closed conductive paths, so induced voltages drive circulating currents (eddy currents). These currents oppose the original magnetic field (Lenz’s law).

Sources of the changing magnetic field include:

- Fast signal edges (high di/dt)

- Switching power stages

- Coil currents

- Return currents changing path with frequency

- Mechanical motion in magnetic fields (rare but real)

2. What eddy currents do in practice

2.1 Power loss and heating

Eddy currents dissipate power as:

P≈B2t2f2ρ

Where:

- B= magnetic flux density

- t= plane thickness

- f= frequency

- ρ= resistivity

At high frequencies or near coils, this becomes measurable heating.

2.2 Ground bounce and noise injection

Eddy currents:

- Modulate local ground potential

- Inject noise into sensitive analog references

- Create apparent “ground movement” even with a solid plane

This is especially problematic in:

- High-impedance analog front ends

- ADC reference grounds

- Sensor return paths

2.3 Magnetic coupling and distortion

In magnetic systems (coils, transformers, beamforming arrays, precision sensors):

- Eddy currents distort the magnetic field

- Reduce effective inductance

- Reduce Q-factor

- Introduce frequency-dependent phase shifts

This matters directly if your system includes:

- Pulsed coils

- HV fast edges

- Precision timing

3. Frequency dependence (critical point)

Eddy currents are strongly frequency-dependent:

| Frequency regime | Ground plane behavior |
| --- | --- |
| DC / low freq | Ideal equipotential |
| Audio–kHz | Mostly benign |
| 10 kHz–1 MHz | Eddy currents become relevant |
| >1 MHz | Current crowding, skin effect, loop formation |

At high frequency:

- Return current follows minimum inductance, not minimum resistance

- Currents hug signal traces

- Unintended loops appear elsewhere in the plane

4. Why solid ground planes can be problematic near coils or fast edges

A solid plane:

- Encourages large-area current loops

- Supports strong eddy currents

- Couples magnetically into nearby conductors

This is why you may observe:

- Extra damping of a transmitter coil

- Unexpected ringing changes

- Apparent loss of energy

- Increased EMI despite “good grounding”

5. Engineering mitigation techniques

5.1 Slotting or segmentation

Strategic slots:

- Break large current loops

- Dramatically reduce eddy currents

- Must be carefully placed to avoid return-path disruption

Rule:
Slot perpendicular to expected eddy current loops, not randomly.

5.2 Controlled return paths

- Keep signal and return close (coplanar / stripline)

- Avoid forcing return current to detour

- Never split planes under fast signals

This localizes currents and reduces large loops.

5.3 Distance from magnetic sources

- Increase spacing between coils and planes

- Use cutouts under inductors or transmitter coils

- Copper pours under coils are often intentionally removed

5.4 Thinner or resistive planes (advanced)

- Thinner copper reduces eddy current magnitude

- Resistive ground layers (rare, specialized)

- Mesh or hatched ground planes in magnetic regions

6. Interaction with microphonics and dielectrics (important link)

Mechanical vibration + magnetic field + ground plane →

- Eddy currents that modulate with motion

- Converted into voltage noise

- Especially visible in high-impedance or precision systems

This can masquerade as:

- Microphonic cable noise

- Dielectric relaxation artifacts

- “Unexplained” low-frequency noise

7. Practical design rules (concise)

- Solid ground plane is best until magnetic fields are strong

- Near coils: remove or slot copper

- Near fast edges: ensure tight return paths

- Sensitive analog ground ≠ noisy power ground

- Do not assume “more copper = better ground”

8. Bottom line

- Eddy currents are a natural consequence of solid conductors + changing magnetic fields

- Ground planes are not electrically passive at high frequency

- In precision or pulsed systems, eddy currents can dominate loss and noise

- Proper segmentation and field-aware layout matter more than plane size

If you want, describe:

- Frequency range

- Presence of coils or fast HV edges

- Board stackup and copper thickness

- Sensitive nodes

I can then assess whether eddy currents are likely limiting your performance and propose a concrete layout strategy.