PCA fundamentals relevant to this ISR (datasheet mapping)

1.1 PCA interrupt sources and flags (PCA0CN)

On C8051F000, the PCA has:

- A 16-bit counter (PCA0H:PCA0L)

- Multiple capture/compare modules (module 0..4)

- Per-module event flags CCF0..CCF4 plus overflow flag CF in PCA0CN

When a module match/capture occurs, hardware sets CCFn, and if enabled, vectors to the PCA interrupt. Importantly:

- CCFn is not automatically cleared; software must clear it.

- CF is not automatically cleared; software must clear it.

Your ISR follows this exactly:

- if (CCF0) { CCF0 = 0; ... }

- if (CCF1) { CCF1 = 0; ... }

- …

- PCA0CN &= ~0x80; clears CF (bit7)

1.2 How a PCA module is configured (PCA0CPMn bitfield)

Each module has PCA0CPMn with these bits:

| Bit | Name | Meaning |
| --- | --- | --- |
| 6 | ECOMn | Comparator enable (required for compare operation) |
| 5 | CAPPn | Capture on rising edge enable |
| 4 | CAPNn | Capture on falling edge enable |
| 3 | MATn | Match enable (sets CCFn on compare match) |
| 2 | TOGn | Toggle CEXn on match |
| 1 | PWMn | PWM mode enable |
| 0 | ECCFn | Interrupt enable for that module’s CCFn |

The datasheet explicitly notes that Software Timer mode is enabled by setting ECOMn and MATn, and then you can load a new compare value inside the ISR to schedule the next “tick.”

1.3 The meaning of the magic constants 0x49 and 0x09

Your code uses:

- PCA0CPM1 = 0x49;

- PCA0CPM3 = 0x49;

- later PCA0CPM1 = 0x09; and PCA0CPM3 = 0x09;

Decode:

0x49 = 0b0100_1001

- bit6 ECOM = 1 (comparator enabled)

- bit3 MAT = 1 (match enabled)

- bit0 ECCF = 1 (interrupt enabled)

So 0x49 = one-shot compare interrupt enabled (classic “software timer event”).

0x09 = 0b0000_1001

- bit3 MAT = 1

- bit0 ECCF = 1

- BUT bit6 ECOM = 0 → comparator function disabled

With ECOM=0, the compare hardware is effectively disabled (no match generation), so this is a practical “disable compare” pattern, consistent with your comment // disable compare.

2) What this ISR is doing at system level (TEM firmware intent)

Your PCA interrupt is acting as the real-time executive for:

- Detecting transmitter switching edges (likely via capture on module 0)

- Scheduling precisely-timed ADC measurements during on-time and off-time

- Sequencing a sign/polarity pattern (signtable) at each cycle

- Running an auxiliary self-test toggling via module 3

The key concept: PCA module 0 is used as the “time origin” marker (capture), while modules 1 and 3 are used as “scheduled events” (compare interrupts).

3) Module-by-module walkthrough of your ISR

3.1 if (CCF0) — “edge event” (capture timestamp of Sync transition)

You read:

current_capture_value = (PCA0CPH0<<8);

current_capture_value += PCA0CPL0;

This reads the module 0 capture register, which in capture mode contains the PCA counter value latched on the configured input edge. Datasheet describes capture operation as latching PCA0H/L into PCA0CPHn/Ln and setting CCFn.

Then:

(A) On-ramp branch: if (!SyncIn) { // Opramp }

Interpretation: a Sync input transition indicates transmitter turning on / ramping current up.

Actions:

- Timestamping first pulse of a data sequence

if (firstindatseq) {

firstindatseq=0;

cstak.hh=hh; cstak.mm=mm; cstak.ss=ss; cstak.zzz=zzz;

LastUpRamp=0;

}

LastUpRamp = current_capture_value;

- The first time in a “series,” you store time-of-day and clear LastUpRamp.

- Then for every upramp you store LastUpRamp = capture_time to later compute on-time length.

- Schedule an on-time measurement using Timer3

NextADC=1;

Nexttimer=0xffff - CurrentMeaTime;

TIMER3=Nexttimer;

TMR3CN = 0x04;  // start Timer 3

This is a classic pattern: preload Timer3 so it overflows after CurrentMeaTime ticks, which then triggers ADC tracking/conversion if ADC is configured to be Timer3-driven (common on Silabs 8051 variants). Even where ADC trigger details vary by device, the Timer3 overflow-based scheduling concept is standard in the family, and your comment // start timer 3 for at lave on time måling matches that intent.

- Increment counters and allow measurement

ntot_counter++;

readytomess=1;

So: CCF0 + Sync indicates ON-edge → immediately arm on-time ADC measurement.

(B) Off branch: else { // sluk }

Interpretation: Sync edge indicates turn-off.

- Compute on-time using last ON capture

if (LastUpRamp) {

cstak.OnPeriode = current_capture_value - LastUpRamp;

update min/max

}

This uses PCA as your high-resolution timebase: on-time = off_edge_time − on_edge_time.

- Series/pulse counting for “moment” completion

if (++nserier >= momenttable[...].n) {

nserier=0;

sidsteskud=1;

firstindatseq=1;

}

This is the “we fired enough shots for this moment; signal data upload/advance.”

- Schedule an off-time measurement using PCA module 1 as a one-shot compare

Nexttimer = current_capture_value + CurrentMeaoffTime;

PCA0CPH1 = (Nexttimer >> 8);

PCA0CPL1 = (Nexttimer & 0xFF);

PCA0CPM1 = 0x49; // enable compare+match+interrupt

NextADC = 3;

readytomess = 1;

This is exactly “Software Timer mode” behavior: you load the compare register to a future time and enable compare/match interrupt.

So: CCF0 + Sync indicates OFF-edge → schedule a delayed off-time ADC measurement via module 1.

3.2 if (CCF1) — “scheduled off-time measurement + sign/polarity progression”

When the PCA counter reaches PCA0CPH1:PCA0CPL1, module 1 match occurs:

- hardware sets CCF1 (PCA0CN.1)

- vectors into ISR if ECCF1 enabled

Your code:

- Clear flag and disable module 1 compare

CCF1 = 0;

PCA0CPM1 = 0x09; // disable compare

As discussed, 0x09 clears ECOM while leaving MAT/ECCF bits; practically disables further match events.

- Select ADC measurement mode for off-time

NextADC = 3; // Off current, voltage, temperature

- Advance sign/polarity pattern

SignState = signtable[CSignbyte++];

switch(SignState) {

case 0: {Tx1Disable=1; Tx0Disable=1;} break;

case 1: TxPol=0; break;

case 2: TxPol=1; break;

}

if (TxPol) cstak.nplus++; else cstak.nminus++;

if (CSignbyte >= signlength) CSignbyte=0;

Meaning:

- You treat signtable[] as a repeating pattern controlling:

  - transmitter enable/disable

  - polarity

- You count how many + and − shots were executed for bookkeeping / QA.

This is a typical TEM sequencing method: polarity alternation plus occasional blanks for baseline.

- Start the ADC conversion when “ready”

if (readytomess)

led2=!led2;

StartADC();

readytomess=0;

Net: CCF1 is the precisely timed “do off-time measurements and update sign state” interrupt.

3.3 if (CCF2) — reserved / magnet signal hook

You clear CCF2 but do nothing else. This is consistent with “placeholder module” for a future compare/capture (your comment says “for Magsignal”).

3.4 if (CCF3) — self-test / LSync toggling using PCA compare

This is another “software timer” implemented with PCA module 3.

- Disable compare and read current compare/capture time

CCF3=0;

PCA0CPM3=0x09;  // disable compare

current_capture_value = (PCA0CPH3<<8) + PCA0CPL3;

- Alternate between two states, scheduling next edge each time

if (SelfTestState) {

SelfTestState=0;

Nexttimer = current_capture_value + OnPeriode;

LSync=0; // Skyder

} else {

SelfTestState=1;

Nexttimer = current_capture_value + OffPeriode;

LSync=1; // Sluk

}

- Program next compare and re-enable module 3 compare interrupt

PCA0CPH3 = Nexttimer >> 8;

PCA0CPL3 = Nexttimer & 0xFF;

PCA0CPM3 = 0x49; // enable compare+match+interrupt

This is canonical PCA “software timer mode”: on each interrupt, add the next interval and reload compare.

Net: CCF3 creates a periodic toggle of LSync with two different durations (OnPeriode/OffPeriode), likely a self-test gating waveform.

4) Why this architecture is strong for TEM transmitter firmware

- Hard timing is in hardware

- Edge timestamps come from capture hardware (module 0).

- Measurement scheduling uses compare hardware (module 1/3).
This avoids software jitter and “instruction path variability.”

- It separates “time marking” from “time scheduling”

- Module 0: tells you what time an external event happened (capture).

- Module 1/3: tells you when a future event must happen (compare).

- On-time vs off-time measurement windows are explicit

- Timer3 is used for on-time measurement delay.

- PCA module 1 for off-time measurement delay from the turn-off edge.

- Deterministic polarity sequencing

- Polarity changes and blanking decisions happen at a known, repeatable point (CCF1 event).

5) Key datasheet-driven correctness points (things this code is doing “right”)

- Clearing CCFn in software: required by the datasheet.

- Clearing CF (overflow) explicitly: required by the datasheet.

- Using ECOM+MAT for software timer compare: called out by the datasheet as enabling software timer behavior.

- Using PCA0MD CPS bits (timebase) matters for absolute time scaling: your timing math assumes a known PCA tick rate set by PCA0MD (SYSCLK/12, SYSCLK/4, Timer0 overflow, or ECI transitions).

6) Two engineering notes (worth checking)

- Compare scheduling across PCA overflow
You are doing:

Nexttimer = current_capture_value + delta;

This is correct in modulo-16-bit arithmetic as long as current_capture_value is truly the PCA counter time reference (it is) and delta is within a sane range. The PCA compare match logic naturally works modulo 0x10000.

- “Disable compare” via 0x09
This works because ECOM=0 disables the comparator function. If you ever see unexpected CCF events, the fully unambiguous disable is PCA0CPMn = 0x00. However, your pattern is consistent with ECOM being the effective gate for match generation.